# EECE.5862 — Embedded Artificial Intelligence

## Lab 1 Report — Microcontroller Programming + First TFLite Micro Inference

| Name | Himadri Saha |
| --- | --- |
| Student ID | 01951522 |
| Course section | EECE.5862 (graduate) |
| Lab | Lab 1: Microcontroller Programming + First TFLite Micro Inference |
| Date submitted | 2026-06-07 |
| Instructor | Prof. Yan Luo |

## 1. Objectives

Lab 1 was a refresher in core embedded skills (digital I/O, debouncing, interrupts), introduced TensorFlow Lite Micro, and explored how embedded ML models interact with the hardware real time. It characterized the model’s interactions with the hardware, by verifying that the model’s output can drive physical interfaces quantifying the recourse costs associated with running the model. Overall, lab 1 explored how resource constrained devices behave under real-time pressure.

## 2. Setup

- D3 → one end of the 220 Ω resistor

- Other end of the 220 Ω resistor → LED anode (long leg)

- LED cathode (short leg) → GND

## 3. Method and Results

Requirement 8.

Measurements: 347,880 bytes flash / 54,048 bytes RAM

Requirement 9.

| Style | Workload | Worst-case response |
| --- | --- | --- |
| Interrupt | delay(1) | 1,050 µs |
| Interrupt | delay(10) | 10,752 µs |
| Interrupt | delay(50) | 50,023 µs |
| Polling | delay(1) | 19 µs |
| Polling | delay(10) | 27 µs |
| Polling | delay(50) | 8 µs |

### 3.1 Part A.1 — Blink

### 3.2 Part A.2 —  Blink + Serial logging

### 3.3 Part A.3 — Polling-based debounced button

### 3.4 Part A.4 — Interrupt-based debounced button

### 3.6 Part B.1 — Annotate hello_world.ino

### 3.7 Part B.2 — Compile & verify hello_world

### 3.8 Part B.3 — Instrument for inference latency

### 3.9 Part B.4 — Reflection paragraphs

### 3.10 Part C.1 — Polling vs. interrupt latency characterization

## 4. Analysis

Requirement 8:

A) When measured against the Nano 33 BLE Sense's actual hardware limits — 1 MB (1,048,576 B) flash and 256 KB (262,144 B) SRAM, ~33% of flash and ~21% of RAM. The actual sine-wave model (g_model byte array) is only on the order of a couple KB. The bulk of the 347 KB is the TFLM interpreter, op resolver, and Mbed OS core, not the model itself. A 10× larger model (~20–30 KB): trivial — easily absorbed by the ~635 KB of flash headroom remaining. 50×–100× (~125–250 KB) larger model might fit, but RAM becomes a constraint first. TFLM needs to hold more tensors for every layers weights and activations at once. As that scales with the model (an image for example), it might blow past the 208KB SRAM making that the real constraint. 
B) A regression example is an accelerometer-driven model maps a window of raw vibration samples which is a continuous output. A classification example is a wake-word detection system, as it maps short audio snippets which are one of a small, fixed set of discrete labels.

Requirement 9.

Polling Style: Worst-case response scales almost linearly with the workload delay. This follows directly from the code structure: the ISR stamps press_timestamp_us at the instant of the physical press (hardware interrupt latency on the nRF52840 is sub-microsecond), but the actual digitalWrite toggle happens later in loop(), only once the blocking delay(WORKLOAD_DELAY_MS) finishes and buttonPressed is re-checked.

Interrupt style: Worst-case responses show no meaningful relationship to the workload. This is because press_timestamp_us is stamped only after the press is already detected and debounced. The measured interval spans just the few microseconds between two micros() calls with a digitalWrite() in between.

## 5. AI Tool Disclosure

| Tool | What you asked it (prompt summary) | What it produced | How you modified / verified it |
| --- | --- | --- | --- |
| Claude | In req9, why is the code not printing the RESP log every time I press the button, even beyond the debounce time | It's gated by kPressBatchSize (50) — recordPressResponse() only calls Serial.println("[RESP] ...") when press_count >= kPressBatchSize (req9.ino:42-54). The per-press feedback you do see every time is the separate "Button pressed (interrupt style) ¦ response = ..." line (req9.ino:91) — [RESP] only fires once every 50 registered presses, then resets the counter to 0. | Used the AI output to brainstorm a starting point – none of the listed issues were the problem |

## 6. References

- Iodice, G. M. TinyML Cookbook (2nd ed.). Packt, 2023.

- Arduino. Nano 33 BLE Sense Rev2 documentation. add the specific pages you used.