Himadri Saha

01951522

Word Classification

2026-06-21

1. Refine Problem Statement

This project implements a real-time keyword spotter on the Arduino Nano 33 BLE Sense Rev2 that classifies spoken color words — "red," "green," and "blue" — and drives three corresponding GPIO-connected LEDs as physical output. The onboard MP34DT06JTR PDM microphone continuously captures 16 kHz PCM audio; a sliding-window MFCC pipeline extracts a 49×40 feature tensor per inference window; an INT8-quantized 2D depthwise-separable CNN (trained via Edge Impulse) performs classification using TensorFlow Lite for Microcontrollers; and an argmax + confidence-threshold decision layer triggers ISR-deferred GPIO writes to the appropriate LED. The system counts as "working" when it correctly illuminates the right LED for each spoken keyword at a false-trigger rate low enough to be visually indistinguishable from correct operation during a live demonstration. Scope has not changed since the Project Ideation post.

2. Dataset Status

Blocked. The audio capture firmware — specifically the PDM-to-PCM pipeline and the sliding-window buffering logic — is not yet finalized, which prevents recording labeled samples directly from the target hardware using Edge Impulse's data forwarder. Additionally, the confidence-threshold and decision logic need to be settled before labeling conventions (including the "unknown/noise" class boundaries) can be locked in. In the meantime, the Google Speech Commands dataset (v2) is being used as a stand-in: it contains pre-recorded 1-second WAV clips for the words "red," "blue," and a superset of color-adjacent words, with a large "unknown" and "background noise" pool available for negative class balancing. This allows the model architecture and training pipeline to be validated end-to-end on the host before any device-side data collection begins. Target sample counts once collection opens: ~150 samples per keyword class (red / green / blue) plus ~150 background/noise samples, collected across at least 3 speakers in varied acoustic conditions.

3. Initial model trained off device

A baseline model has been trained on the Speech Commands stand-in data using Edge Impulse's Keras export path with the following configuration:

- Architecture: Conv2D(8, 3×3, relu) → DepthwiseConv2D(3×3, relu) → Conv2D(16, 1×1, relu) → GlobalAvgPool → Dense(4, softmax) — a minimal DS-CNN targeting the 49×40 MFE input shape

- Parameter count: ~3,160 (INT8-quantized, matching Lab 2 Track K baseline)

- Held-out accuracy (float32 validation set): 85.65%

- INT8 post-training quantization accuracy: 85.99% (quantization did not degrade accuracy on this architecture)

The INT8 .tflite model has been successfully converted to a C array and compiled against TFLM on the host; a mock inference pass with a zeroed input tensor completes without error, confirming the op resolver and tensor arena are correctly configured.

4. Blockers

The ring buffer and PDM callback design currently used in the Arduino PDM library does not expose a clean way to detect mid-word silence boundaries, making it unclear how to reliably segment continuous speech into 1-second inference windows without either clipping word onsets or accumulating too much pre-silence padding — what is the recommended windowing strategy for continuous KWS on the Nano 33 BLE Sense when the PDM callback fires asynchronously relative to the inference loop?